TRANSFER STATION

CONSTRUCTION SPECIFICATIONS

Phase I – Cast in Place Concrete

The Town of Monterey, Massachusetts

Department of Public Works

2016

Bid Package, Town of Monterey

Transfer Station Facility

This bid package along with the plans labeled “Proposed Transfer Station” constitutes the entire bid package.  The following general requirements will apply.

- All bidders shall provide a resume of public work similar to the work proposed by this bid.

- Proof of liability insurance and workman’s compensation insurance must accompany bid.

- A certificate of Non-Collusion must accompany bid.

- All on site personnel must be OSHA Safety Certified.

- The contractor selected will be responsible for obtaining a building permit and all building permit fees will be waived as this is a Town of Monterey project, and as such shall be reflected in the bid price.

- A 50% payment bond will be required of the successful bidder within 10 days of Notice of Award.

- The contractor must comply will all applicable Federal, State and Local laws, statutes and rules and regulations, including discrimination in employment.  The Town of Monterey encourages minority and women in employment on this project.

- Job to be started by May 1, 2016.  Any changes must be approved in writing by the Director of Operations for the Town of Monterey.

- A payment schedule shall be provided by the successful bidder within 10 days of notice of award and be approved by the Town of Monterey

- Any change orders shall be in writing and approved by the Director of Operations.

DIVISION I – GENERAL REQUIREMENTS

SECTION 1 – Summary of Work

1.1	SCOPE OF WORK

The following shall be a part of all Divisions and Sections of the Work and shall be as binding as if written at the head of each Division or Section:

The Contractor shall provide all items, articles, materials, operations, tools, equipment, labor, and incidentals necessary and required to complete in an acceptable manner all work under the above Heading, as indicated, implied, mentioned, shown, or scheduled on the Drawings, and/or as specified herein, or both.

All parts of the "Instructions to Bidders", "General Conditions", "Supplementary General Conditions", and "Division I—General Requirements" shall be made a part of this Heading and shall be as binding as if herein written.

Materials normally delivered labeled shall be received with manufacturer's original label and instructions, or else shall be subject to rejection.  Materials shall be stored under adequately clean and dry conditions, and all work shall be performed according to the best practice of the trades.  Manufacturer's specifications and instructions for products specified herein or approved equals, become part of these specifications and all such instructions are to be followed accordingly.

All work shall be completed in compliance with the latest edition of the prevailing fire prevention and building codes in effect in the State of Massachusetts.

1.2	OUTLINE OF WORK

The work under this Contract shall consist of, but shall not necessarily be limited to, the following:

A.		Cast in Place Concrete for the new transfer station.

DIVISION I – GENERAL REQUIREMENTS

Section 2 – Submittals

1.1	GENERAL

A.	All required submittals shall be made to the Owner for review by the Engineer.

B.	Submittals (such as brochures, schedules, performance charts, etc.) that show more than one item or piece of equipment shall be clearly marked to show which item is being submitted for approval.

C.	Each submittal shall bear the following:

1.	Owner's name

2.	Project name and address

3.	Contractor's/Subcontractor's name

4.	Manufacturer's or Supplier's name

D.	Any work which is the subject of a required submittal shall be commenced only after approval of the submittal has been obtained.

1.2	PRODUCT DATA

A.	List of materials and quantities.

DIVISION II – SITEWORK AND PAVING

2.1	GENERAL

A.	Related Documents

All Work under this division shall be performed in accordance with the Drawings and the provisions of the Contract, including General Conditions, Supplementary General Conditions, and Division I—General Requirements of these Specifications.

B.	Scope of Work

Owner and Contractor shall coordinate the Work of this division so as to avoid delays to the greatest extent possible.

1.	Work to be done by Owner

a.	Excavation and Filling

The Owner shall perform all excavation and filling, including that required for the foundations.

b.	Site Grading and Drainage

The Owner shall grade the site so that it is level to plus or minus 0.1 foot inside the building area.  The Owner shall be responsible for all site drainage, including grading of the area surrounding the building so that slope away from the building is adequate to insure proper drainage.  The Owner shall provide for all final grading and site restoration.

c.	Paving

The Owner will be responsible for all paving.

2.	Work to be done by Contractor

- The Owner shall excavate and backfill to original grade for all foundation work.  The Contractor shall have no responsibility for work more than ten feet (10') outside the building wall or lean-to foundations.  Contractor to pull all necessary permits.

- Optional galvanized railing, concrete to be dropped 3 foot in elevation.

2.2	PROTECTION

A.	Owner and Contractor shall provide complete surface protection of their respective portions of the Work so as to protect both workers and the general public, and shall do so in complete accordance with local regulations.

B.	All surface drainage shall be diverted during construction in such a manner as to avoid damage to the site or any adjacent area.

2.3	EXECUTION

A.	Owner shall excavate to lines and grades indicated on the drawings for all footings.

B.	No foundation work shall begin until the Engineer or the Owner's designated representative has approved the excavations.

C.	No backfilling shall be done until:

1.	the foundations have been inspected and approved by the Engineer or the Owner's designated representative.

D.	Compact fill material in lifts not exceeding 6" to 95% of its Proctor density.

DIVISION III – CAST-IN-PLACE CONCRETE

3.1	GENERAL

A.	Related Documents

All Work under this division shall be performed in accordance with the Drawings and the provisions of the Contract, including General Conditions, Supplementary General Conditions, and Division I—General Requirements of these Specifications.

B.	Scope of Work

Work under this division shall consist of concrete footings and walls as shown on Drawings.

C.	Quality Assurance

Concrete work shall be in accordance with current design plans enclosed with bid request packet

Concrete supplier is to provide recent documentation showing mix design and results verifying the mix design will produce 4000 PSI within 28 days. Sample testing must be completed from plant in which the concrete will be supplied from.

The town retains the right to hire a third party consultant to perform testing on concrete supplied to the project.

Exposed concrete shall be trowel finished.

Concrete slabs shall be saw-cut to create control joints in accordance with general control joint practices and shall be verified with engineer.

D.	Submittals

1.	Refer to Division I, Section 2, Submittals.

2.	Material Certificate:  submit certificate from concrete supplier, showing compliance with design requirements for strength.

3.	Product Data:  submit data for proprietary materials and items, including reinforcement and forming accessories, admixtures, and others as requested by Engineer or Owner's designated representative.

B I D

Proposal of 	                                                                            , hereinafter called Bidder, organized and existing under the laws of the State of 	                                       , doing business as a/an (corporation) (partnership) (individual) [circle one], and if a  corporation, authorized to do business in the State of Massachusetts,

To the Town of Monterey, Massachusetts, hereinafter called Owner:

The undersigned has carefully examined the Plans, Specifications, and other Contract Documents, has familiarized himself with the site and other conditions affecting the Work, and acknowledges receipt of the following Addendum/a if any:

In compliance with your Advertisement for Bids, Bidder hereby proposes to perform all Work for the construction of a new transfer station facility in strict accordance with the Contract Documents, within the time set forth therein, and at the prices stated below.

By submission of this Bid, the Bidder certifies, and in the case of a joint Bid each party thereto certifies as to his own organization, that this Bid has been arrived at independently, without consultation, communication, or agreement as to any matter relating to this Bid with any other Bidder or with any competitor.

In submitting this Bid, Bidder agrees that this Bid will remain in effect for a period of forty-five (45) calendar days following the opening of the Bids, and that the undersigned agrees to enter into a Contract with the Owner, if awarded, on the basis of this Bid.

Bidder hereby agrees to commence Work under this Contract or on before a date to be specified in the Notice to Proceed and to fully complete the Project not later than ninety (90) calendar days thereafter.

Bidder agrees to perform all the Work described in the Contract Documents for the following amount:

Lump Sum Base Bid:

(written)

(figures)

Attached to this Proposal is a Bid Bond in the following amount, which represents not less than five (5) percent of the amount of the Base Bid:

Amount of Bid Bond:

(written)

(figures)

Respectfully submitted:

Firm Name

and Address:

Authorized Signature:

Title:

License Number

if applicable:

Date                        	Telephone (        )

SEAL (if Bid is by a corporation)

Attest

REFERENCE LIST

Bidder shall submit information on this sheet indicating construction experience on similar work.  Failure to complete this sheet may be cause for rejection of bid.

1.	Type of Structure	________________________________________________

Year Constructed	________________________________________________

Owner	________________________________________________

Location (City, State)	________________________________________________

Owner's Contact Person	________________________________________________

Owner's Telephone No.	________________________________________________

2.	Type of Structure	________________________________________________

Year Constructed	________________________________________________

Owner	________________________________________________

Location (City, State)	________________________________________________

Owner's Contact Person	________________________________________________

Owner's Telephone No.	________________________________________________

3.	Type of Structure	________________________________________________

Year Constructed	________________________________________________

Owner	________________________________________________

Location (City, State)	________________________________________________

Owner's Contact Person	________________________________________________

Owner's Telephone No.	________________________________________________

4.	Type of Structure	________________________________________________

Year Constructed	________________________________________________

Owner	________________________________________________

Location (City, State)	________________________________________________

Owner's Contact Person	________________________________________________

Owner's Telephone No.	________________________________________________

CERTIFICATE OF NON-COLLUSION

The undersigned certifies under penalties of perjury that this bid or proposal has been made and submitted in good faith and without collusion or fraud with any other person.  As used in this certification, the word “person” shall mean any natural person, business, partnership, corporation, union, committee, club, or other organization, entity, or group of individuals.

_______________________________________

(Signature of individual submitting bid or proposal)

_______________________________________

(Name of business)